#include <EEPROM.h>

#include <LiquidCrystal_I2C.h>

#include <SoftwareSerial.h>

// ================== CONFIG ==================

#define HALL_PIN 3

#define GSM_TX 11

#define GSM_RX 10

const float WHEEL_RADIUS_M = 0.3;

unsigned long SERVICE_INTERVAL = 5000;   // Default, can be changed via SMS

const unsigned long SAVE_INTERVAL = 100;

#define MILEAGE_ADDR 0

#define SERVICE_FLAG_ADDR 10

#define SERVICE_INTERVAL_ADDR 20   // New: Save service interval in EEPROM

String ADMIN_PASSWORD = "123456789";

String authorizedPhone = "+255612267910";  // Main phone

LiquidCrystal_I2C lcd(0x27, 16, 2);

SoftwareSerial gsm(GSM_RX, GSM_TX);

volatile unsigned long pulseCount = 0;

unsigned long totalPulses = 0;

float totalKm = 0.0;

unsigned long lastSavePulses = 0;

unsigned long lastSaveTime = 0;

bool serviceAlert = false;

String smsBuffer = "";

bool adminMode = false;

float getPulsesPerKm() {

float circumference = 2 * PI * WHEEL_RADIUS_M;

return 1000.0 / circumference;

}

void pulseISR() {

pulseCount++;

}

void setup() {

Serial.begin(9600);

gsm.begin(9600);

pinMode(HALL_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(HALL_PIN), pulseISR, RISING);

lcd.init();

lcd.backlight();

lcd.print("Mileage System");

delay(2000);

lcd.clear();

// Load saved data

EEPROM.get(MILEAGE_ADDR, totalPulses);

EEPROM.get(SERVICE_FLAG_ADDR, serviceAlert);

EEPROM.get(SERVICE_INTERVAL_ADDR, SERVICE_INTERVAL);

totalKm = totalPulses / getPulsesPerKm();

lastSavePulses = totalPulses;

Serial.println("System Started");

updateLCD();

initGSM();

}

void loop() {

noInterrupts();

unsigned long currentPulses = pulseCount;

pulseCount = 0;

interrupts();

if (currentPulses > 0) {

totalPulses += currentPulses;

totalKm = totalPulses / getPulsesPerKm();

checkService();

updateLCD();

if ((totalPulses - lastSavePulses) >= SAVE_INTERVAL || millis() - lastSaveTime > 10000) {

saveToEEPROM();

}

}

checkSMS();

delay(100);

}

void updateLCD() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Mileage: ");

lcd.print(totalKm, 1);

lcd.print(" km");

if (serviceAlert) {

lcd.setCursor(0, 1);

lcd.print("SERVICE DUE!");

} else {

lcd.setCursor(0, 1);

lcd.print("Ready");

}

}

void checkService() {

unsigned long kmInt = (unsigned long)totalKm;

if (!serviceAlert && kmInt >= SERVICE_INTERVAL && (kmInt % SERVICE_INTERVAL == 0)) {

serviceAlert = true;

EEPROM.put(SERVICE_FLAG_ADDR, serviceAlert);

sendSMS("Service due! Mileage: " + String(totalKm, 1) + " km. Reply 'done'");

updateLCD();

}

}

void saveToEEPROM() {

EEPROM.put(MILEAGE_ADDR, totalPulses);

EEPROM.put(SERVICE_INTERVAL_ADDR, SERVICE_INTERVAL);

lastSavePulses = totalPulses;

lastSaveTime = millis();

}

// ================== GSM FUNCTIONS ==================

void initGSM() {

delay(1000);

gsm.println("AT");

delay(500);

gsm.println("AT+CMGF=1");

delay(500);

gsm.println("AT+CNMI=1,2,0,0,0");

delay(500);

Serial.println("GSM Ready");

}

void sendSMS(String msg) {

gsm.print("AT+CMGS=\"" + authorizedPhone + "\"");

gsm.print("\r");

delay(100);

gsm.print(msg);

gsm.write(26);

delay(1500);

Serial.println("SMS Sent");

}

// ================== SMS MENU & CONTROL ==================

void checkSMS() {

while (gsm.available()) {

char c = gsm.read();

smsBuffer += c;

}

if (smsBuffer.length() > 0) {

String msg = smsBuffer;

msg.toLowerCase();

String originalMsg = smsBuffer;

if (msg.indexOf("menu") != -1) {

sendSMS("Commands:\nmenu\npassword 123456789\nset service XXXX\nadd phone +255...");

}

else if (msg.indexOf("password " + ADMIN_PASSWORD) != -1) {

adminMode = true;

sendSMS("Admin access granted!");

}

else if (adminMode && msg.indexOf("set service") != -1) {

int val = originalMsg.substring(originalMsg.indexOf("service") + 8).toInt();

if (val > 1000) {

SERVICE_INTERVAL = val;

EEPROM.put(SERVICE_INTERVAL_ADDR, SERVICE_INTERVAL);

sendSMS("Service interval updated to " + String(SERVICE_INTERVAL) + " km");

}

}

else if (adminMode && msg.indexOf("add phone") != -1) {

int start = originalMsg.indexOf("+");

if (start != -1) {

authorizedPhone = originalMsg.substring(start, start + 13);

sendSMS("New phone added: " + authorizedPhone);

}

}

else if (msg.indexOf("done") != -1) {

serviceAlert = false;

EEPROM.put(SERVICE_FLAG_ADDR, serviceAlert);

updateLCD();

sendSMS("Service alert cleared.");

}

else if (msg.indexOf("mileage") != -1 || msg.indexOf("status") != -1) {

sendSMS("Current Mileage: " + String(totalKm, 1) + " km");

}

smsBuffer = "";

// Auto logout admin after 1 command

if (adminMode) adminMode = false;

}

}